CMI NaaP AI Network Intelligence

Blast Radius Analysis — AMDL Alignment

Project 01 | AMDL Alignment

Prepared by: Alpit Tare

Date: March 22, 2026 | Version: 1.0

# Blast Radius Analysis - AMDL Alignment

## AMDL Framework Overview

AMDL (Add/Modify/Delete/Lookup) is the core operational lifecycle model for CMI NaaP. Every network change follows one of these four operations, and Blast Radius Analysis provides intelligent support for all of them.

## ADD Operation - Service Provisioning Simulation

### Purpose

Enable safe, predictable addition of new services, configurations, or network resources while validating that:

- Resource availability and capacity exist

- Dependencies can be satisfied

- No policy violations occur

- High availability/redundancy requirements are met

### Blast Radius Analysis Support

### Simulation of New Service Provisioning

Use Case: Add new data center interconnect (DCI) service
 
1. Proposed Change Input:
   {
     "operation": "ADD",
     "service": "dci-nyc-lax",
     "type": "mpls_vpn",
     "bandwidth_gbps": 10,
     "latency_sla_ms": 50,
     "availability_requirement": "99.99%",
     "required_devices": ["nyc-core-1", "lax-core-1"],
     "redundancy_requirement": "dual_path"
   }
 
2. Analysis Pipeline:
   a. Check resource availability
      - Are required devices available?
      - Do they have sufficient bandwidth?
      - Are they in required geographic locations?
 
   b. Validate redundancy
      - Are there dual independent paths?
      - Do paths avoid single points of failure?
      - Can service survive device failure?
 
   c. Dependency verification
      - Does DCI depend on routing services?
      - Are routing services available on required devices?
      - Can required services handle new traffic load?
 
   d. Conflict detection
      - Would VLANs conflict with existing allocations?
      - Are IP ranges already in use?
      - Does service fit within existing network design?
 
   e. Risk assessment
      - Configuration complexity: 18/20 (complex BGP/MPLS setup)
      - Blast radius: 24 affected services (dependent services)
      - Dependency depth: 5 levels
      - Overall risk: 55/100 (Medium)
 
3. Output Report:
   {
     "operation": "ADD",
     "feasibility": {
       "can_provision": true,
       "violations": [],
       "warnings": [
         "BGP sessions will need reconfiguration on 4 devices",
         "MPLS LSP creation may cause brief traffic impact (< 100ms)"
       ]
     },
     "impact_analysis": {
       "affected_services": 24,
       "affected_devices": 12,
       "estimated_provisioning_time": "2 hours",
       "estimated_traffic_impact": "100ms latency spike during provisioning"
     },
     "dependencies": {
       "required_services": [
         "bgp_routing", "mpls_forwarding", "device_mgmt"
       ],
       "configuration_dependencies": [
         "bgp_asn_allocation",
         "vlan_creation",
         "ip_allocation"
       ]
     },
     "resource_check": {
       "bandwidth_available": "25 Gbps (sufficient)",
       "device_capacity": "Operating at 45% (sufficient)",
       "vlan_availability": "150 free VLANs (sufficient)"
     }
   }
 
4. Recommendations:
   - Schedule provisioning during maintenance window (service not yet active)
   - Pre-stage BGP configurations on both core devices
   - Have rollback plan ready (removal of service config)
   - Validate SLA requirements are met post-provisioning

### Scenario-Based Impact Analysis

Scenario 1: What if NYC core device fails during provisioning?
- Service would not be operational (dependency on that device)
- Recommendation: Only provision to devices with N+1 redundancy
 
Scenario 2: What if MPLS label space exhausted?
- Service provisioning would fail
- Recommendation: Check MPLS label allocation before provisioning
 
Scenario 3: What if BGP AS path loops back to sender?
- Service would create routing loops
- Recommendation: Validate BGP design against routing policy

### ADD Operation AMDL Details

| Aspect | Details |
| --- | --- |
| Operation Type | CREATE - New network object/service |
| Change Scope | Service, Device, Configuration, Virtual Network |
| Validation Focus | Resource availability, conflicts, dependencies |
| Risk Factors | Complexity, blast radius, dependency depth |
| Key Metrics | Resource utilization, VLAN availability, capacity |
| Simulation Type | Constraint satisfaction, resource allocation |
| Rollback | Delete/remove the added service |
| Approval Gates | Risk score > 50 requires approval |
| Audit Trail | What was added, by whom, when, risk assessment |

---

## MODIFY Operation - Configuration Change Analysis

### Purpose

Analyze the impact of modifying existing network configurations, ensuring that:

- Changes don't break existing service dependencies

- Configuration changes are backward compatible

- Service SLAs are maintained

- Policy compliance is preserved

### Blast Radius Analysis Support

### Configuration Modification Impact Analysis

Use Case: Increase BGP route-refresh timer from 60 to 120 seconds
 
1. Proposed Change Input:
   {
     "operation": "MODIFY",
     "scope": "bgp_routing_service",
     "device": "core-1",
     "change_type": "configuration",
     "modification": {
       "parameter": "route_refresh_timer",
       "old_value": 60,
       "new_value": 120,
       "affected_peers": ["core-2", "edge-1", "edge-2"]
     }
   }
 
2. Analysis Pipeline:
   a. Dependency tracing
      - Which services depend on BGP routing?
      - What is SLA impact of 120s refresh interval?
      - Which customers are affected?
 
   b. Topology impact
      - How many devices have route-refresh timers?
      - Are they all compatible with 120s interval?
      - Does this affect convergence time?
 
   c. Transitive impact
      - BGP affects MPLS which affects VPN services
      - 5 customer VPNs use BGP routing
      - 12,000 end users indirectly affected
 
   d. Risk calculation
      - Criticality: 25/30 (BGP is critical)
      - Blast radius: 12/25 (12 VPN services affected)
      - Complexity: 3/20 (simple config change)
      - Dependency depth: 9/15 (3 levels deep)
      - Total risk: 49/100 (Medium)
 
3. Output Report:
   {
     "operation": "MODIFY",
     "change_id": "CHG-20240321-001",
     "feasibility": {
       "can_deploy": true,
       "validation_errors": [],
       "warnings": [
         "Route convergence time will increase to ~60-90 seconds",
         "Temporary packet loss during BGP flap: < 1 second"
       ]
     },
     "affected_items": {
       "services": [
         {
           "id": "vpn_customer_a",
           "name": "Customer A VPN",
           "criticality": "high",
           "impact": "longer_convergence_time"
         },
         {
           "id": "vpn_customer_b",
           "name": "Customer B VPN",
           "criticality": "high",
           "impact": "longer_convergence_time"
         }
       ],
       "devices": [
         {
           "id": "core-2",
           "name": "Core Switch 2",
           "impact": "route_refresh_changes"
         }
       ]
     },
     "sla_impact": {
       "convergence_time": {
         "before_ms": 30,
         "after_ms": 75,
         "sla_requirement_ms": 100,
         "compliant": true
       },
       "availability": {
         "estimated_impact": "< 1 second outage during convergence",
         "sla_requirement": "99.99%",
         "compliant": true
       }
     },
     "peer_compatibility": {
       "compatible_peers": ["core-2", "edge-1", "edge-2"],
       "incompatible_peers": [],
       "requires_peer_coordination": true
     }
   }
 
4. Implementation Plan:
   - Stage 1: Update configuration template
   - Stage 2: Deploy to non-critical edge devices first
   - Stage 3: Wait 30 minutes, monitor for issues
   - Stage 4: Deploy to core-1 and core-2
   - Stage 5: Rollback configuration (revert to 60s timer)

### Dependency Chain Analysis

Change: Modify OSPF cost on core-1 -> edge-1 link
 
Service Impact Chain:
  edge-1 device
    └─ interfaces (physical layer)
         └─ OSPF routing (protocol layer)
              └─ BGP redistribution (protocol)
                   └─ customer VPNs (service layer)
                        └─ end users (application)
 
Questions Answered:
1. Which OSPF routes are affected? (Path engineering routes)
2. Will BGP prefer alternate paths? (Yes, if available)
3. Which VPNs use these routes? (3 customer VPNs)
4. What's the traffic shift? (15% to alternate path)
5. Can alternate path support load? (Yes, 70% utilization)
6. What's the convergence time? (2-5 seconds)
7. Will any customer SLA be violated? (No)

### MODIFY Operation AMDL Details

| Aspect | Details |
| --- | --- |
| Operation Type | UPDATE - Modify existing configuration |
| Change Scope | Configuration parameters, service attributes |
| Validation Focus | Backward compatibility, SLA compliance, policy |
| Risk Factors | Blast radius, dependency depth, change complexity |
| Key Metrics | Affected services, convergence time, capacity impact |
| Simulation Type | Configuration application, impact propagation |
| Rollback | Revert to previous configuration version |
| Approval Gates | Risk score > 50 requires approval |
| Audit Trail | What changed, old/new values, affected items |

---

## DELETE Operation - Removal Impact Evaluation

### Purpose

Safely decommission or remove network services and configurations while:

- Ensuring no orphaned dependencies remain

- Identifying services that will be affected

- Planning cleanup of related configurations

- Maintaining service continuity during removal

### Blast Radius Analysis Support

### Service Removal Impact Analysis

Use Case: Remove legacy MPLS VPN service (customer contract ended)
 
1. Proposed Change Input:
   {
     "operation": "DELETE",
     "scope": "service",
     "service_id": "mpls_vpn_legacy_customer_x",
     "service_name": "Legacy Customer X VPN",
     "removal_reason": "contract_ended",
     "dependencies_action": "plan_removal"
   }
 
2. Analysis Pipeline:
   a. Dependency discovery
      - What other services depend on this VPN?
      - Are there any critical dependencies?
      - Can dependents survive removal?
 
   b. Orphan detection
      - Dedicated infrastructure for this service?
      - Shared infrastructure with other services?
      - VLANs, IP subnets used only by this service?
 
   c. Impact scope
      - Which devices would be affected?
      - How many configurations need removal?
      - Are there backup/standby instances?
 
   d. Cascading impact
      - Other services using this VPN? (None identified)
      - Monitoring/alerting on this service? (Yes, 5 alerts)
      - Billing systems tracking this? (Yes, need to update)
 
   e. Risk assessment
      - Risk of affecting other services: Low
      - Risk of leaving orphaned config: Medium
      - Overall risk: 25/100 (Low)
 
3. Output Report:
   {
     "operation": "DELETE",
     "removal_feasible": true,
     "can_remove_immediately": false,
     "validation_errors": [],
     "warnings": [
       "Customer may lose connectivity if not migrated to new service",
       "Monitoring alerts configured for this service - need cleanup",
       "VLAN 250 is dedicated to this service - will become unused"
     ],
     "dependencies": {
       "services_depending_on_this": [],
       "infrastructure_shared_with": [
         "core_mgmt_vpn",
         "monitoring_network"
       ],
       "dedicated_infrastructure": [
         "vlan_250",
         "bgp_as_path_segment_65001"
       ]
     },
     "cleanup_tasks": [
       {
         "priority": "high",
         "task": "Remove VLAN 250 configuration",
         "affected_devices": ["core-1", "core-2", "edge-1"],
         "estimated_time_minutes": 15
       },
       {
         "priority": "high",
         "task": "Remove BGP configuration for customer",
         "affected_devices": ["core-1", "core-2"],
         "estimated_time_minutes": 10
       },
       {
         "priority": "medium",
         "task": "Update monitoring system",
         "affected_devices": ["monitoring_server"],
         "estimated_time_minutes": 5
       },
       {
         "priority": "low",
         "task": "Update billing system",
         "affected_systems": ["billing"],
         "estimated_time_minutes": 5
       }
     ],
     "removal_plan": {
       "pre_removal_validation": [
         "Confirm customer has migrated to new service",
         "Verify no active traffic on VPN"
       ],
       "removal_sequence": [
         "Stop BGP route advertisements (controlled graceful shutdown)",
         "Wait 5 minutes for traffic to reroute",
         "Remove BGP configuration",
         "Remove VLAN 250 from all devices",
         "Remove monitoring alerts",
         "Update billing system"
       ],
       "rollback_capability": {
         "rollback_possible": true,
         "rollback_time_minutes": 20,
         "rollback_data_available": true
       }
     }
   }
 
4. Implementation Plan:
   - Pre-removal: Contact customer, confirm migration
   - Removal window: 30 minutes during maintenance window
   - Sequence: Stop BGP → Wait 5min → Config removal → Cleanup
   - Validation: Confirm VLAN not in use, monitoring alerts cleared
   - Rollback: Have config backup ready for 30 days

### Orphan Configuration Detection

Orphan Detection Algorithm:
 
For service being removed:
1. Find all infrastructure components (devices, VLANs, IPs)
2. For each component:
   a. Check if other services use this component
   b. If no other services, mark as "potential orphan"
   c. Check if component is explicitly reusable
   d. If reusable, add to cleanup list
   e. If not reusable, flag for manual review
 
Results:
- Dedicated to removed service: VLAN 250, BGP AS path
- Shared infrastructure: MPLS core (used by other VPNs)
- Reusable after cleanup: IP address block 10.100.0.0/16

### DELETE Operation AMDL Details

| Aspect | Details |
| --- | --- |
| Operation Type | DESTROY/DECOMMISSION - Remove network object |
| Change Scope | Service, Device, Configuration, Virtual Network |
| Validation Focus | Dependency safety, orphan detection, cleanup |
| Risk Factors | Blast radius of removal, orphaned config |
| Key Metrics | Affected services, cleanup tasks, rollback time |
| Simulation Type | Dependency impact, cleanup validation |
| Rollback | Recreate/restore removed components |
| Approval Gates | Manual review required (no auto-approval) |
| Audit Trail | What was removed, by whom, reason, cleanup status |

---

## LOOKUP Operation - Topology & Dependency Retrieval

### Purpose

Enable engineers to query and understand current network state, including:

- Current topology snapshots

- Service dependency graphs

- Device/service relationships

- Historical changes and evolution

### Blast Radius Analysis Support

### Real-Time Topology Queries

Use Case 1: "Show me all services that depend on core-1 device"
 
Query:
{
  "operation": "LOOKUP",
  "query_type": "dependency_reverse",
  "scope": "device",
  "device_id": "core-1",
  "include_indirect": true
}
 
Response:
{
  "query_id": "qry-uuid",
  "timestamp": "2024-03-21T10:15:30Z",
  "results": {
    "direct_dependencies": [
      {
        "service_id": "bgp_routing",
        "service_name": "BGP Routing",
        "criticality": "critical",
        "dependency_type": "strong",
        "impact_if_failed": "All routing stops"
      }
    ],
    "indirect_dependencies": [
      {
        "service_id": "vpn_customer_a",
        "service_name": "Customer A VPN",
        "criticality": "high",
        "dependency_path": "bgp_routing -> vpn_customer_a",
        "impact_if_core1_fails": "VPN convergence time 60+ seconds"
      }
    ],
    "affected_users": 50000,
    "total_affected_services": 12
  }
}

### Dependency Graph Visualization

Use Case 2: "Show me the complete dependency graph for customer_vpn_x"
 
Query:
{
  "operation": "LOOKUP",
  "query_type": "dependency_graph",
  "scope": "service",
  "service_id": "customer_vpn_x",
  "graph_direction": "both",  // upstream and downstream
  "include_topology": true
}
 
Response:
{
  "graph": {
    "nodes": [
      {
        "id": "customer_vpn_x",
        "type": "service",
        "criticality": "high"
      },
      {
        "id": "bgp_routing",
        "type": "service",
        "criticality": "critical"
      },
      {
        "id": "core-1",
        "type": "device",
        "status": "operational"
      },
      {
        "id": "vpn_customer_x_user_1",
        "type": "application",
        "users": 50
      }
    ],
    "edges": [
      {
        "source": "customer_vpn_x",
        "target": "bgp_routing",
        "relationship": "DEPENDS_ON",
        "criticality": "strong"
      },
      {
        "source": "bgp_routing",
        "target": "core-1",
        "relationship": "RUNS_ON",
        "criticality": "strong"
      },
      {
        "source": "vpn_customer_x_user_1",
        "target": "customer_vpn_x",
        "relationship": "USES",
        "criticality": "strong"
      }
    ]
  },
  "statistics": {
    "total_nodes": 150,
    "total_edges": 300,
    "depth": 6,
    "critical_nodes": 5
  }
}

### Historical Change Queries

Use Case 3: "Show me all changes to core-1 in the last 30 days"
 
Query:
{
  "operation": "LOOKUP",
  "query_type": "change_history",
  "scope": "device",
  "device_id": "core-1",
  "time_range": {
    "start": "2024-02-20T00:00:00Z",
    "end": "2024-03-21T23:59:59Z"
  }
}
 
Response:
{
  "changes": [
    {
      "change_id": "CHG-20240315-001",
      "timestamp": "2024-03-15T14:30:00Z",
      "change_type": "MODIFY",
      "description": "Increased MTU from 1500 to 9000",
      "risk_score": 42,
      "status": "deployed",
      "affected_services": 8,
      "applied_by": "engineer@company.com"
    },
    {
      "change_id": "CHG-20240310-003",
      "timestamp": "2024-03-10T09:15:00Z",
      "change_type": "MODIFY",
      "description": "Updated BGP timer configuration",
      "risk_score": 28,
      "status": "deployed",
      "affected_services": 12,
      "applied_by": "engineer@company.com"
    }
  ],
  "summary": {
    "total_changes": 7,
    "successful": 7,
    "failed": 0,
    "average_risk_score": 35,
    "services_affected_total": 45
  }
}

### LOOKUP Operation AMDL Details

| Aspect | Details |
| --- | --- |
| Operation Type | READ - Query network state |
| Query Types | Topology, dependencies, history, analytics |
| Response Time | <500ms for typical queries |
| Cache Usage | Heavily cached (1 hour TTL) |
| Authorization | Read-only access |
| Audit Trail | Query logged with timestamp and user |
| Performance | Optimized with indexes and materialized views |

---

## AMDL Integration Matrix

### Cross-Operation Dependencies

ADD        MODIFY     DELETE     LOOKUP
       ─────────────────────────────────────────
ADD    Conflict   ADD then   Not        LOOKUP
       Check      MODIFY     allowed    prep
       ─────────────────────────────────────────
MODIFY LOOKUP     Conflict   Not        LOOKUP
       first      Check      allowed    first
       ─────────────────────────────────────────
DELETE LOOKUP     Not        Prep with  LOOKUP
       first      allowed    LOOKUP     prep
       ─────────────────────────────────────────
LOOKUP Safe       Safe       Safe       Safe
       read-only  read-only  read-only  read-only

### Typical Workflow Sequences

### Workflow 1: Add New Service (ADD)

1. LOOKUP: Query current topology and available resources
2. Simulate ADD: Test in cloned environment
3. Risk Assessment: Score the change (target: < 50)
4. Engineer Review: Approve/modify
5. Implementation: Execute ADD
6. LOOKUP: Verify service added correctly

### Workflow 2: Update Configuration (MODIFY)

1. LOOKUP: Get current service dependencies
2. Simulate MODIFY: Test configuration in cloned environment
3. Impact Analysis: Calculate blast radius
4. Risk Assessment: Score the change
5. Engineer Review: Peer review recommended if risk > 50
6. Implementation: Execute MODIFY with rollback ready
7. LOOKUP: Verify service still healthy

### Workflow 3: Decommission Service (DELETE)

1. LOOKUP: Identify all dependencies and orphaned infrastructure
2. Simulation: Plan removal sequence
3. Impact Analysis: Ensure no critical breakage
4. Cleanup Planning: Detail all cleanup tasks
5. Manual Approval: Required for DELETE operations
6. Implementation: Execute removal in planned sequence
7. Validation: Confirm cleanup complete
8. LOOKUP: Verify dependencies updated

## Compliance & Audit

### AMDL Compliance Tracking

Every operation logged with full context:

{
  "audit_record": {
    "timestamp": "2024-03-21T10:15:30Z",
    "user": "engineer@company.com",
    "operation": "MODIFY",
    "change_id": "CHG-20240321-001",
    "change_scope": "bgp_configuration",
    "analysis_results": {
      "blast_radius": 24,
      "risk_score": 42,
      "risk_level": "medium",
      "approval_required": false
    },
    "approval_status": "approved",
    "approved_by": "operator@company.com",
    "implementation": {
      "start_time": "2024-03-21T10:20:00Z",
      "end_time": "2024-03-21T10:22:30Z",
      "status": "success",
      "duration_seconds": 150
    },
    "validation": {
      "post_change_check": "passed",
      "sla_impact": "within_tolerance",
      "rollback_required": false
    }
  }
}

## Success Metrics for AMDL Alignment

| Metric | Target | Current | Gap |
| --- | --- | --- | --- |
| ADD operations with pre-validation | 100% | TBD | - |
| MODIFY operations analyzed | 100% | TBD | - |
| DELETE operations with cleanup plan | 100% | TBD | - |
| LOOKUP response time | <500ms | TBD | - |
| Risk scoring accuracy | >90% | TBD | - |
| False positive rate | <5% | TBD | - |
| Approval gate effectiveness | 95% | TBD | - |