CMI NaaP AI Network Intelligence

Self-Healing Network

High-Level Design

Project 03 | HLD

Prepared by: Alpit Tare

Date: March 21, 2026 | Version: 1.0

# Self-Healing Network - High-Level Design

## Overview

Self-Healing Network (SHN) is a production-grade, closed-loop automation system for network fault detection, diagnosis, and remediation. It combines ML-driven anomaly detection with policy-based remediation and safety guardrails.

Priority: High | Status: Requires Uplift

Missing Components (Designed, Require Uplift for Production): Closed-loop automation framework, Policy engine, Safe execution guardrails

Business Value: Reduced Downtime, Automated Recovery, Operational Resilience

## Architecture

┌─────────────────────────────────────────────────────────────────────┐
│                      Telemetry Collection Layer                     │
│                    (SNMP, NetFlow, syslog, APIs)                   │
└─────────────────┬───────────────────────────────────────────────────┘
                  │
┌─────────────────v───────────────────────────────────────────────────┐
│                  Self-Healing Network Pipeline                      │
├─────────────────────────────────────────────────────────────────────┤
│                                                                     │
│  ┌──────────┐  ┌──────────┐  ┌────────┐  ┌────────┐  ┌──────────┐ │
│  │ DETECT   │→ │ ANALYZE  │→ │DECIDE  │→ │ ACT    │→ │ VERIFY   │ │
│  │          │  │          │  │        │  │        │  │          │ │
│  │ Anomaly  │  │ML Model  │  │Policy  │  │Remedy  │  │Confirm   │ │
│  │Detection │  │Class.    │  │Engine  │  │Execute │  │Effective │ │
│  └──────────┘  └──────────┘  └────────┘  └────────┘  └──────────┘ │
│                                                                     │
└─────────────────────────────────────────────────────────────────────┘
         ↑                                                    ↓
         └────────────── Guardrails Layer ──────────────────┘
                (Rate Limiting, Rollback, Change Windows)

### Architecture Diagram (State Machine & Component Flow)

stateDiagram-v2
    [*] --> IDLE
    IDLE --> DETECTING: New Metrics
    DETECTING --> ANALYZING: Anomaly Detected
    ANALYZING --> DECIDING: Classification Complete
    DECIDING --> ACTING: Policy Approved
    DECIDING --> IDLE: No Action Needed
    ACTING --> VERIFYING: Remediation Executed
    VERIFYING --> IDLE: Success & Verified
    VERIFYING --> ROLLING_BACK: Health Degraded
    ROLLING_BACK --> IDLE: Rollback Complete
    ACTING --> FAILED: Execution Error
    DECIDING --> FAILED: Policy Blocked
    FAILED --> [*]
 
    note right of DETECTING
        FaultDetector:
        - Z-Score Detection
        - EWMA Drift Detection
        - Threshold-Based Detection
    end note
 
    note right of ANALYZING
        FaultClassifier (8 classes)
        HealthScorer (5-component)
        FailurePredictor
    end note
 
    note right of DECIDING
        PolicyEngine (6 policies)
        GuardrailManager
    end note
 
    note right of ACTING
        RemediationExecutor
        Command Validator
    end note
 
    note right of VERIFYING
        RemediationVerifier
        Rollback Trigger
    end note

Component Dependency Graph

Config Layer (settings.py + policies.yaml)
    │
    ├─→ FaultDetector (Z-Score, EWMA, Threshold)
    │       │
    │       └─→ FaultClassifier (8 fault classes)
    │
    ├─→ HealthScorer (5-component assessment)
    │
    ├─→ FailurePredictor (1h/4h/24h forecasting)
    │
    ├─→ PolicyEngine (6 policies)
    │
    └─→ GuardrailManager
            │
            └─→ RemediationExecutor
                    │
                    └─→ RemediationVerifier

## Technology Stack

- Statistical Analysis: Pure NumPy for Z-score, EWMA, logistic regression

- Data Structures: Python dataclasses for type-safe, lightweight models

- GuardrailCheck: Rate limiting, change window, rollback conditions

- RemediationAction: Command, device, metadata

- ExecutionResult: Status, output, timestamp

- VerificationResult: Health delta, success/failure flags

- HealingEvent: Complete lifecycle record

- HealingReport: Aggregated metrics and outcomes

- Configuration: YAML-based policy definitions + Python settings module

- Logging: Python logging framework with structured event capture

## Key Components

### 1. Detection Layer

- Z-Score Anomaly Detection: Statistical anomaly detection based on standard deviations

- EWMA Drift Detection: Exponential Weighted Moving Average for trend detection

- Threshold-Based Detection: Simple threshold crossing for known metrics

### 2. Analysis Layer

- Fault Classifier: Decision tree classifier identifying fault type (5 classes)

- Health Scorer: Weighted multi-metric health assessment

- Failure Predictor: Logistic regression predicting 1h/4h/24h failure probability

### 3. Decision Layer

- Policy Engine: Rule-based remediation policy evaluation

- Guardrail Manager: Safety checks and rate limiting

- Priority Ranking: Highest priority remediation first

### 4. Action Layer

- Remediation Generator: Creates Cisco CLI commands

- Command Validator: Safety checks before execution

- Execution Manager: Simulated device interaction

### 5. Verification Layer

- Post-Remediation Metrics: Re-collect metrics after action

- Effectiveness Verification: Measure improvement

- Rollback Trigger: Automatic revert if situation worsens

## Data Model

### Metric Schema

The system monitors the following device metrics for anomaly detection:

cpu_percent: CPU utilization (0-100)
memory_percent: Memory utilization (0-100)
interface_errors: CRC/input/output error count
packet_loss_percent: Percentage of dropped packets
bgp_neighbors_down: Count of down BGP sessions
bgp_neighbors_up: Count of up BGP sessions
stp_blocked_ports: Number of STP-blocked ports
link_utilization_percent: Interface utilization (0-100)

### HealingEvent Dataclass

@dataclass
class HealingEvent:
    timestamp: datetime
    device_id: str
    fault_type: str  # e.g., "BGP_FLAP", "LINK_DOWN"
    severity: str    # "CRITICAL", "WARNING", "INFO"
    anomaly_score: float
    health_score: float
    remediation_action: Optional[RemediationAction]
    execution_result: Optional[ExecutionResult]
    verification_result: Optional[VerificationResult]
    rollback_triggered: bool
    state_transitions: List[str]

### Policy Condition Format

policies:
  bgp_flap_recovery:
    conditions:
      - metric: bgp_neighbors_down
        operator: ">"
        threshold: 0
        duration_seconds: 30
    actions:
      - command: "clear ip bgp * soft"
    guardrails:
      rate_limit: 5_per_hour
      change_window: "02:00-06:00 UTC"

## Supported Devices

- Nexus 9000 (NX-OS)

- Catalyst 6509 (IOS)

## Supported Fault Types

- Link Down: Interface not passing traffic

- BGP Flap: BGP session instability

- Interface Flap: Interface up/down cycling

- High CPU: CPU utilization above threshold

- STP Loop: Spanning Tree Protocol loop detected

- Link Errors: CRC/input/output errors

- Packet Loss: Data loss on link

## Generated Cisco Commands

- interface X shutdown / no shutdown

- clear ip bgp X.X.X.X soft in

- clear ip bgp * soft

- spanning-tree vlan X priority Y

- switchport trunk allowed vlan add X

## Telemetry Simulator

The system includes a synthetic telemetry generator for testing and demonstration without requiring physical devices.

### Features

- Synthetic Metric Generation: Baseline metrics with configurable normal range

- Anomaly Injection: Programmatic fault injection for testing detection accuracy

- CPU spikes (threshold crossing)

- BGP session flaps (metric oscillation)

- Link errors (sudden error count increase)

- Packet loss (sustained degradation)

- Device Simulation: Emulates real device behavior

- Nexus 9000 (NX-OS): BGP, STP, interface metrics

- Catalyst 6509 (IOS): Interface state, route table size, CPU/memory

- Time-Series Patterns: Incorporates day-of-week and hour-of-day variations

### Usage Example

from simulator import TelemetrySimulator, AnomalyInjection
 
sim = TelemetrySimulator(device_type="nexus_9000")
injection = AnomalyInjection(
    fault_type="bgp_flap",
    start_time="2024-01-01 10:00:00",
    duration_seconds=300
)
metrics = sim.generate_metrics(duration=600, anomalies=[injection])

## Safety Mechanisms

- Rate Limiter: Max 5 remediation actions per hour

- Rollback Trigger: Revert if health degrades >10%

- Change Window: Only remediate 02:00-06:00 UTC by default

- Emergency Override: Can temporarily disable guardrails

## Data Flow

Device Metrics
     ↓
Telemetry Simulator / SNMP Collection
     ↓
Anomaly Detection (Z-score + EWMA + Threshold)
     ↓
Fault Classification (Decision Tree)
     ↓
Health Scoring (Weighted Multi-metric)
     ↓
Failure Prediction (Logistic Regression)
     ↓
Policy Evaluation
     ↓
Guardrail Check
     ↓
Remediation Execution (if approved)
     ↓
Post-Remediation Verification
     ↓
Audit Log / Metrics Storage

## State Machine States

- IDLE: Waiting for metrics

- DETECTING: Analyzing for anomalies

- ANALYZING: Classifying fault type

- DECIDING: Evaluating policies and guardrails

- ACTING: Executing remediation

- VERIFYING: Checking post-remediation metrics

- ROLLING_BACK: Reverting failed remediation

- FAILED: Unable to remediate

## Configuration

- YAML-based policy definitions

- Python-based settings

- Environment-based overrides

## Performance Metrics

- Detection Latency: <1 second

- Classification Accuracy: ~85% (on simulated data)

- Remediation Success Rate: 80-90% (depends on fault type)

- Average Recovery Time: 30-120 seconds per fault

## Integration Points

- REST API for metrics ingestion

- Webhook support for notifications

- CLI for manual trigger

- Streamlit dashboard for visualization

- Kubernetes deployment support

## Implementation Status

### Completed Components

- Anomaly detection framework (Z-Score, EWMA, Threshold)

- Fault classification (8 classes via decision tree)

- Health scoring (5-component weighted assessment)

- Failure predictor (logistic regression for 1h/4h/24h)

- State machine implementation (8 states with transitions)

- Guardrail manager (rate limiting, change windows)

- Remediation executor (Cisco CLI command generation)

- Verification layer (post-remediation effectiveness checks)

- Telemetry simulator (synthetic metric generation with anomaly injection)

- YAML-based policy engine (6 configurable policies)

- Python dataclass models (GuardrailCheck, RemediationAction, etc.)

- Logging and audit trail infrastructure

### Future Work

- LLM integration for root cause analysis

- RAG engine with runbook knowledge base

- Multi-device correlation analysis

- Predictive maintenance scheduling

- Integration with existing NMS/SIEM systems

- Advanced ML models (LSTM, XGBoost)

- Time-series database optimization

- High-availability deployment patterns

- Webhook-based escalation system