High-Level Design (HLD)

InterAS Option B — MPLS VPN Connectivity

Cisco Nexus 9000 Series | NX-OS MPLS

AS 65001 (E01/E02) ↔ AS 65000 (R01/R02 VPE)

Port-Channel Sub-Interface | FW Contexts | VRF CMI | Primary/Secondary

March 2026 | Version 3.0

# 1. Executive Summary

This High-Level Design defines the MPLS VPN InterAS Option B architecture connecting AS 65001 (E01 and E02 Nexus devices) to AS 65000 (R01 and R02 VPE routers) for CMI service connectivity. Customer VRFs on E01/E02 leak routes to cust_out, egress through dedicated firewall contexts, pass ACLs into VRF CMI, and reach the peer AS over port-channel sub-interfaces via MP-eBGP Option B with RFC 3107.

The inter-AS connectivity uses Port-channel1 on both E01 and E02, with per-customer dot1q sub-interfaces providing VLAN-level traffic segregation toward VPE routers. E01/R01 serves as the primary path; E02/R02 as secondary.

# 2. Scope and Objectives

## 2.1 Scope

- AS 65001: E01 and E02 Nexus devices (same AS) with iBGP VPNv4/v6 between them

- AS 65000: R01 and R02 VPE routers providing CMI service access

- InterAS Option B (RFC 3107) over Port-channel1 sub-interfaces: E01↔R01 (primary), E02↔R02 (secondary)

- Per-customer dot1q sub-interfaces on port-channel for VLAN-segregated traffic toward VPE

- Customer VRFs: Cust A, Cust B (E01), Cust C (E02) with VRF route leaking to cust_out

- Firewall integration: FW Context A (E01 path), FW Context B (E02 path)

- VRF CMI on E01/E02 as post-firewall transit VRF toward peer AS

## 2.2 Objectives

- Primary/secondary redundant MPLS VPN connectivity between AS 65001 and AS 65000

- Per-customer VLAN segregation on port-channel toward VPE for traffic isolation and QoS

- Centralized firewall policy enforcement per path (FW Context A and B)

- Complete IGP isolation between ASes using RFC 3107 BGP labeled unicast

# 3. Targeted Design Architecture

## 3.1 Design Overview

E01 and E02 in AS 65001 serve as both PE and ASBR. Customer VRFs leak routes into cust_out, which forwards through dedicated firewall contexts. Post-firewall traffic enters VRF CMI, which reaches VPE routers R01/R02 (AS 65000) via port-channel1 sub-interfaces. Each customer gets a dedicated dot1q-tagged sub-interface on the port-channel, providing VLAN-level segregation on the inter-AS link.

## 3.2 Network Topology

## 3.3 Port-Channel Sub-Interface Design

The inter-AS link between E01/E02 and R01/R02 uses a port-channel (LACP) with per-customer dot1q sub-interfaces. This provides link aggregation for bandwidth and redundancy, while VLAN tagging ensures per-customer traffic isolation on the wire.

| Device | Port-Channel | Sub-Interface | VLAN (dot1q) | VRF | Customer |
| --- | --- | --- | --- | --- | --- |
| E01 | Po1 | Po1.100 | 100 | CMI | Cust A |
| E01 | Po1 | Po1.200 | 200 | CMI | Cust B |
| E02 | Po1 | Po1.300 | 300 | CMI | Cust C |

## 3.4 Traffic Flow — Primary Path (E01 → R01)

Customer A/B VRF (E01) → route leak via RT to cust_out VRF → FW Context A (out→in) → ACL → VRF CMI (E01) → Port-channel1 sub-interface (dot1q tagged per customer) → MP-eBGP Option B → R01 (AS 65000) → VRF CMI → CMI Services

## 3.5 Traffic Flow — Secondary Path (E02 → R02)

Customer C VRF (E02) → route leak via RT to cust_out VRF → FW Context B (out→in) → ACL → VRF CMI (E02) → Port-channel1.300 (dot1q 300) → MP-eBGP Option B → R02 (AS 65000) → VRF CMI → CMI Services

## 3.6 Device Role Summary

| Device | AS | Role | VRFs | InterAS Link | Path |
| --- | --- | --- | --- | --- | --- |
| E01 | 65001 | PE/ASBR | Cust_A/B, cust_out, CMI | Po1 (.100,.200) → R01 | Primary |
| E02 | 65001 | PE/ASBR | Cust_C, cust_out, CMI | Po1 (.300) → R02 | Secondary |
| R01 | 65000 | VPE | CMI | Po1 (.100,.200) ← E01 | Primary |
| R02 | 65000 | VPE | CMI | Po1 (.300) ← E02 | Secondary |

## 3.7 VRF Architecture

### 3.7.1 VRF Design on E01

| VRF | RD | RT Import | RT Export | Purpose |
| --- | --- | --- | --- | --- |
| Cust_A | 65001:100 | 65001:100, 65001:999 | 65001:100 | Customer A traffic |
| Cust_B | 65001:200 | 65001:200, 65001:999 | 65001:200 | Customer B traffic |
| cust_out | 65001:999 | 65001:100, 65001:200 | 65001:999 | FW egress aggregation |
| CMI | 65001:500 | 65001:500, 65000:500 | 65001:500 | Post-FW transit (Po1 sub-ints) |

### 3.7.2 VRF Design on E02

| VRF | RD | RT Import | RT Export | Purpose |
| --- | --- | --- | --- | --- |
| Cust_C | 65001:300 | 65001:300, 65001:999 | 65001:300 | Customer C traffic |
| cust_out | 65001:999 | 65001:300 | 65001:999 | FW egress aggregation |
| CMI | 65001:500 | 65001:500, 65000:500 | 65001:500 | Post-FW transit (Po1 sub-ints) |

## 3.8 Route Leaking Logic

- Stage 1 — Customer to cust_out: Customer VRFs export with unique RTs, cust_out imports all customer RTs

- Stage 2 — cust_out to Firewall: cust_out default route points to FW (Context A on E01, Context B on E02)

- Stage 3 — Firewall to VRF CMI: Post-FW traffic enters VRF CMI, forwarded via Po1 sub-interfaces to VPE

- Return: cust_out exports RT 65001:999 (default via FW), imported by all customer VRFs

## 3.9 Firewall Integration

| FW Context | Device (Primary/Sec) | Outside VRF | Inside VRF |
| --- | --- | --- | --- |
| FW Context A | E01 (Primary) | cust_out | CMI |
| FW Context B | E02 (Secondary) | cust_out | CMI |

# 4. InterAS Option B Design

## 4.1 Why Option B with RFC 3107

- Single MP-eBGP session per ASBR pair carries all VPN prefixes

- RFC 3107 eliminates LDP/IGP between ASes — complete control plane isolation

- Port-channel provides link aggregation (bandwidth + member-level redundancy)

- Per-customer sub-interfaces enable VLAN-segregated traffic and per-customer QoS/policing

- NSF and Graceful Restart for hitless failover

## 4.2 BGP Session Architecture

| Type | From | To | Address Family | Purpose |
| --- | --- | --- | --- | --- |
| iBGP | E01 | E02 | VPNv4/v6 Unicast | Intra-AS VPN route sync |
| eBGP | E01 | R01 | VPNv4 + Labeled-Unicast | Primary InterAS (over Po1 sub-ints) |
| eBGP | E02 | R02 | VPNv4 + Labeled-Unicast | Secondary InterAS (over Po1 sub-ints) |

## 4.3 MPLS Label Stack

- Outer label (IGP/Core): Directs packet to ASBR (via BGP labeled-unicast / RFC 3107)

- Inner label (VPN): Directs packet to correct VRF on destination device

## 4.4 Primary/Secondary Failover

Normal operation: traffic via E01 → Po1 sub-ints → R01. If E01↔R01 port-channel goes down (all members fail), BGP withdraws routes and traffic shifts to E02 → Po1 sub-ints → R02. iBGP between E01/E02 ensures full VPN table on both devices. Port-channel member-level failures are handled by LACP without BGP reconvergence.

# 5. Prerequisites

- MPLS license installed on E01 and E02 Nexus devices

- NX-OS version 7.2(0)D1(1)+ for full Option B / RFC 3107

- Port-channel member interfaces cabled between E01↔R01 and E02↔R02

- LACP configured on both ends of each port-channel

- Firewall contexts (A and B) provisioned with inside/outside interfaces

- VLAN IDs (100, 200, 300) coordinated with VPE team for sub-interface tagging

# 6. Design Recommendations

- Use LACP active mode on both ends for fast failure detection

- Configure lacp min-links to ensure minimum bandwidth before declaring port-channel up

- Apply per-sub-interface QoS policies for customer-level traffic shaping toward VPE

- Use send-community both on all eBGP VPNv4/v6 sessions

- Apply LDP-filtering ACLs (port 646) on port-channel sub-interfaces for RFC 3107

- Use next-hop-self on iBGP VPNv4/v6 between E01 and E02

- Test failover by shutting individual port-channel members and full port-channel

# 7. Verification Commands

| Command | Purpose |
| --- | --- |
| show port-channel summary | Verify Po1 state and member interfaces |
| show interface port-channel1 | Port-channel status and traffic stats |
| show interface Po1.100 | Sub-interface status per customer VLAN |
| show bgp vpnv4 unicast | Display VPN routes from BGP table |
| show bgp ipv4 labeled-unicast | Verify RFC 3107 label exchange |
| show ip route vrf CMI | Verify CMI VRF routes toward peer AS |
| show ip route vrf cust_out | Verify cust_out routing and FW default |
| show vrf | Verify all VRF RD and interface assignments |

Refer to the Low-Level Design (LLD) v3.0 for device-specific configurations.