Low-Level Design (LLD)

InterAS Option B — Device Configuration

Cisco Nexus 9000 | Port-Channel Sub-Interface Design

E01/E02 (AS 65001) ↔ R01/R02 (AS 65000)

Po1 Sub-Interfaces | VRF Leaking | FW Contexts | RFC 3107

March 2026 | Version 3.0

# 1. IP Addressing Scheme

## 1.1 Loopback Addresses

| Device | Loopback0 | AS | Role |
| --- | --- | --- | --- |
| E01 | 1.1.1.1/32 | 65001 | PE/ASBR (Primary) |
| E02 | 2.2.2.2/32 | 65001 | PE/ASBR (Secondary) |
| R01 | 3.3.3.3/32 | 65000 | VPE (Primary) |
| R02 | 4.4.4.4/32 | 65000 | VPE (Secondary) |

## 1.2 Port-Channel and Sub-Interface Addressing

| Device | Interface | VLAN | IP Address | VRF | Peer | Customer |
| --- | --- | --- | --- | --- | --- | --- |
| E01 | Po1.100 | 100 | 10.100.1.1/30 | CMI | R01 Po1.100 | Cust A |
| E01 | Po1.200 | 200 | 10.200.1.1/30 | CMI | R01 Po1.200 | Cust B |
| R01 | Po1.100 | 100 | 10.100.1.2/30 | CMI | E01 Po1.100 | Cust A |
| R01 | Po1.200 | 200 | 10.200.1.2/30 | CMI | E01 Po1.200 | Cust B |
| E02 | Po1.300 | 300 | 10.300.1.1/30 | CMI | R02 Po1.300 | Cust C |
| R02 | Po1.300 | 300 | 10.300.1.2/30 | CMI | E02 Po1.300 | Cust C |

## 1.3 iBGP Peering Link

| Link | E01 IP | E02 IP |
| --- | --- | --- |
| E01 ↔ E02 (iBGP) | 10.0.0.1/30 | 10.0.0.2/30 |

## 1.4 Port-Channel Member Interfaces

| Device | Port-Channel | Member Interfaces | LACP Mode |
| --- | --- | --- | --- |
| E01 | Po1 | Eth1/1, Eth1/2 | Active |
| R01 | Po1 | Eth1/1, Eth1/2 | Active |
| E02 | Po1 | Eth1/1, Eth1/2 | Active |
| R02 | Po1 | Eth1/1, Eth1/2 | Active |

## 1.5 VRF Allocation Summary

| VRF | Device | RD | RT Import | RT Export | Purpose |
| --- | --- | --- | --- | --- | --- |
| Cust_A | E01 | 65001:100 | 65001:100, 65001:999 | 65001:100 | Customer A |
| Cust_B | E01 | 65001:200 | 65001:200, 65001:999 | 65001:200 | Customer B |
| Cust_C | E02 | 65001:300 | 65001:300, 65001:999 | 65001:300 | Customer C |
| cust_out | E01,E02 | 65001:999 | 65001:100,200,300 | 65001:999 | FW egress |
| CMI | E01,E02 | 65001:500 | 65001:500, 65000:500 | 65001:500 | Post-FW to VPE |

# 2. E01 Configuration (AS 65001 — Primary ASBR)

## 2.1 Feature Enablement

configure terminal

install feature-set mpls

feature-set mpls

feature mpls ldp

feature mpls l3vpn

feature bgp

feature lacp

feature interface-vlan

## 2.2 Port-Channel and Member Interfaces

interface Ethernet1/1

description Po1 Member - To R01 (AS 65000)

channel-group 1 mode active

no shutdown

!

interface Ethernet1/2

description Po1 Member - To R01 (AS 65000)

channel-group 1 mode active

no shutdown

!

interface port-channel1

description To R01 (AS 65000) - Primary InterAS Option B

switchport

switchport mode trunk

switchport trunk allowed vlan 100,200

lacp min-links 1

mtu 9216

no shutdown

## 2.3 Port-Channel Sub-Interfaces (Per-Customer)

interface port-channel1.100

description Cust A toward R01 (VRF CMI)

encapsulation dot1q 100

vrf member CMI

ip address 10.100.1.1/30

mpls ip

no shutdown

!

interface port-channel1.200

description Cust B toward R01 (VRF CMI)

encapsulation dot1q 200

vrf member CMI

ip address 10.200.1.1/30

mpls ip

no shutdown

## 2.4 Other Interfaces

interface loopback0

ip address 1.1.1.1/32

!

interface Ethernet1/3

description To E02 - iBGP Peering

ip address 10.0.0.1/30

no shutdown

!

interface Ethernet1/4

description To FW Context A - Outside (cust_out)

vrf member cust_out

ip address <FW_CTXA_OUTSIDE_E01_IP>/<MASK>

no shutdown

!

interface Ethernet1/5

description From FW Context A - Inside (VRF CMI)

vrf member CMI

ip address <FW_CTXA_INSIDE_E01_IP>/<MASK>

no shutdown

!

interface Ethernet1/6

description Customer A CE

vrf member Cust_A

ip address <CUST_A_LINK_IP>/<MASK>

no shutdown

!

interface Ethernet1/7

description Customer B CE

vrf member Cust_B

ip address <CUST_B_LINK_IP>/<MASK>

no shutdown

## 2.5 VRF Configuration

### 2.5.1 Customer A VRF

vrf context Cust_A

rd 65001:100

address-family ipv4 unicast

route-target import 65001:100

route-target import 65001:999

route-target export 65001:100

### 2.5.2 Customer B VRF

vrf context Cust_B

rd 65001:200

address-family ipv4 unicast

route-target import 65001:200

route-target import 65001:999

route-target export 65001:200

### 2.5.3 cust_out VRF

vrf context cust_out

rd 65001:999

address-family ipv4 unicast

route-target import 65001:100

route-target import 65001:200

route-target import 65001:300

route-target export 65001:999

### 2.5.4 VRF CMI

vrf context CMI

rd 65001:500

address-family ipv4 unicast

route-target import 65001:500

route-target import 65000:500

route-target export 65001:500

## 2.6 BGP Configuration

### 2.6.1 Global + eBGP to R01 (RFC 3107) via Po1 Sub-Interfaces

router bgp 65001

address-family ipv4 unicast

redistribute direct route-map LOOPBACK

allocate-label all

!

! eBGP to R01 via Po1.100 - Labeled Unicast (RFC 3107)

neighbor 10.100.1.2

remote-as 65000

address-family ipv4 labeled-unicast

!

! eBGP to R01 - VPNv4 via loopback

neighbor 3.3.3.3

remote-as 65000

update-source loopback0

ebgp-multihop 2

address-family vpnv4 unicast

send-community both

### 2.6.2 iBGP to E02

neighbor 2.2.2.2

remote-as 65001

update-source loopback0

address-family vpnv4 unicast

next-hop-self

send-community both

### 2.6.3 VRF Address Families

vrf Cust_A

address-family ipv4 unicast

vrf Cust_B

address-family ipv4 unicast

vrf cust_out

address-family ipv4 unicast

vrf CMI

address-family ipv4 unicast

## 2.7 Static Routes

! Default route in cust_out → FW Context A outside

vrf context cust_out

ip route 0.0.0.0/0 <FW_CTXA_OUTSIDE_NEXTHOP>

!

! Return path: VRF CMI → FW Context A inside

vrf context CMI

ip route 0.0.0.0/0 <FW_CTXA_INSIDE_NEXTHOP>

## 2.8 LDP Filter ACL (RFC 3107)

ip access-list LDP_FILTER

10 deny tcp any any eq 646

20 deny tcp any eq 646 any

30 deny udp any any eq 646

40 deny udp any eq 646 any

50 permit ip any any

!

interface port-channel1.100

ip access-group LDP_FILTER in

ip access-group LDP_FILTER out

!

interface port-channel1.200

ip access-group LDP_FILTER in

ip access-group LDP_FILTER out

## 2.9 CMI Filter ACL

ip access-list CMI_FILTER

10 permit ip <ALLOWED_NETWORKS> any

20 deny ip any any log

!

interface Ethernet1/5

ip access-group CMI_FILTER in

# 3. E02 Configuration (AS 65001 — Secondary ASBR)

## 3.1 Feature Enablement

configure terminal

install feature-set mpls

feature-set mpls

feature mpls ldp

feature mpls l3vpn

feature bgp

feature lacp

feature interface-vlan

## 3.2 Port-Channel and Member Interfaces

interface Ethernet1/1

description Po1 Member - To R02 (AS 65000)

channel-group 1 mode active

no shutdown

!

interface Ethernet1/2

description Po1 Member - To R02 (AS 65000)

channel-group 1 mode active

no shutdown

!

interface port-channel1

description To R02 (AS 65000) - Secondary InterAS Option B

switchport

switchport mode trunk

switchport trunk allowed vlan 300

lacp min-links 1

mtu 9216

no shutdown

## 3.3 Port-Channel Sub-Interface (Per-Customer)

interface port-channel1.300

description Cust C toward R02 (VRF CMI)

encapsulation dot1q 300

vrf member CMI

ip address 10.300.1.1/30

mpls ip

no shutdown

## 3.4 Other Interfaces

interface loopback0

ip address 2.2.2.2/32

!

interface Ethernet1/3

description To E01 - iBGP Peering

ip address 10.0.0.2/30

no shutdown

!

interface Ethernet1/4

description To FW Context B - Outside (cust_out)

vrf member cust_out

ip address <FW_CTXB_OUTSIDE_E02_IP>/<MASK>

no shutdown

!

interface Ethernet1/5

description From FW Context B - Inside (VRF CMI)

vrf member CMI

ip address <FW_CTXB_INSIDE_E02_IP>/<MASK>

no shutdown

!

interface Ethernet1/6

description Customer C CE

vrf member Cust_C

ip address <CUST_C_LINK_IP>/<MASK>

no shutdown

## 3.5 VRF Configuration

### 3.5.1 Customer C VRF

vrf context Cust_C

rd 65001:300

address-family ipv4 unicast

route-target import 65001:300

route-target import 65001:999

route-target export 65001:300

### 3.5.2 cust_out VRF

vrf context cust_out

rd 65001:999

address-family ipv4 unicast

route-target import 65001:100

route-target import 65001:200

route-target import 65001:300

route-target export 65001:999

### 3.5.3 VRF CMI

vrf context CMI

rd 65001:500

address-family ipv4 unicast

route-target import 65001:500

route-target import 65000:500

route-target export 65001:500

## 3.6 BGP Configuration

### 3.6.1 Global + eBGP to R02 (RFC 3107) via Po1 Sub-Interface

router bgp 65001

address-family ipv4 unicast

redistribute direct route-map LOOPBACK

allocate-label all

!

! eBGP to R02 via Po1.300 - Labeled Unicast (RFC 3107)

neighbor 10.300.1.2

remote-as 65000

address-family ipv4 labeled-unicast

!

! eBGP to R02 - VPNv4 via loopback

neighbor 4.4.4.4

remote-as 65000

update-source loopback0

ebgp-multihop 2

address-family vpnv4 unicast

send-community both

### 3.6.2 iBGP to E01

neighbor 1.1.1.1

remote-as 65001

update-source loopback0

address-family vpnv4 unicast

next-hop-self

send-community both

### 3.6.3 VRF Address Families

vrf Cust_C

address-family ipv4 unicast

vrf cust_out

address-family ipv4 unicast

vrf CMI

address-family ipv4 unicast

## 3.7 Static Routes

vrf context cust_out

ip route 0.0.0.0/0 <FW_CTXB_OUTSIDE_NEXTHOP>

!

vrf context CMI

ip route 0.0.0.0/0 <FW_CTXB_INSIDE_NEXTHOP>

## 3.8 LDP Filter ACL

ip access-list LDP_FILTER

10 deny tcp any any eq 646

20 deny tcp any eq 646 any

30 deny udp any any eq 646

40 deny udp any eq 646 any

50 permit ip any any

!

interface port-channel1.300

ip access-group LDP_FILTER in

ip access-group LDP_FILTER out

## 3.9 CMI Filter ACL

ip access-list CMI_FILTER

10 permit ip <ALLOWED_NETWORKS> any

20 deny ip any any log

!

interface Ethernet1/5

ip access-group CMI_FILTER in

# 4. Route Leaking and Traffic Flow

## 4.1 Route-Target Flow Matrix

| Source VRF | Exports RT | Imported By | On Device | Effect |
| --- | --- | --- | --- | --- |
| Cust_A | 65001:100 | cust_out | E01 (+E02 via iBGP) | Cust A routes → FW |
| Cust_B | 65001:200 | cust_out | E01 (+E02 via iBGP) | Cust B routes → FW |
| Cust_C | 65001:300 | cust_out | E02 (+E01 via iBGP) | Cust C routes → FW |
| cust_out | 65001:999 | All Cust VRFs | All | Default (FW) → customers |
| CMI | 65001:500 | CMI (peer AS) | E01↔R01, E02↔R02 | Post-FW to VPE via Po1 |

## 4.2 End-to-End Path (Primary — Customer A)

- 1. Cust A traffic enters Cust_A VRF on E01

- 2. Matches default route leaked from cust_out (RT 65001:999)

- 3. Forwarded to cust_out VRF → next-hop FW Context A outside

- 4. FW Context A inspects → forwards to inside interface

- 5. CMI_FILTER ACL applied on VRF CMI inside interface

- 6. Traffic enters VRF CMI on E01

- 7. VRF CMI forwards via Po1.100 (dot1q 100) to R01

- 8. R01 (AS 65000) delivers to CMI Services

# 5. Verification Procedure

## 5.1 Commands

show port-channel summary

show interface port-channel1

show interface port-channel1.100

show interface port-channel1.200

show interface port-channel1.300

show lacp counters

show bgp vpnv4 unicast

show bgp vpnv4 unicast neighbors

show bgp ipv4 labeled-unicast

show ip route vrf Cust_A

show ip route vrf Cust_B

show ip route vrf Cust_C

show ip route vrf cust_out

show ip route vrf CMI

show vrf

show mpls forwarding-table vrf CMI

show running-config bgp

## 5.2 Expected Results

| Check | Expected |
| --- | --- |
| show port-channel summary | Po1 up (U), members bundled (P) |
| show interface Po1.100 on E01 | Up/up, vrf CMI, dot1q 100 |
| iBGP E01↔E02 | Established, VPNv4 routes exchanged |
| eBGP E01↔R01 (via Po1 sub-int) | Established, VPNv4 + labeled-unicast |
| show ip route vrf cust_out (E01) | Default via FW + all customer routes |
| show ip route vrf CMI (E01) | Routes from R01 via Po1 sub-ints |
| Failover: shut all Po1 members on E01 | Traffic shifts to E02→R02 |
| Failover: shut 1 Po1 member on E01 | LACP handles, no BGP reconvergence |

# 6. Rollback Procedure

- Pre-change: checkpoint create <name> on both E01 and E02

- Rollback: rollback running-config checkpoint <name>

- Port-channel issue: verify member interfaces, LACP mode, and min-links

- Sub-interface issue: verify dot1q VLAN matches on both ends (E01↔R01, E02↔R02)

- BGP issue: verify neighbor IP matches Po1 sub-interface IP, remote-as, send-community

- VRF route leak issue: check RT import/export match against Section 4.1 matrix

# 7. Placeholders to Complete

| Placeholder | Description |
| --- | --- |
| <FW_CTXA_OUTSIDE_E01_IP>/<MASK> | E01 interface to FW Context A outside |
| <FW_CTXA_INSIDE_E01_IP>/<MASK> | E01 interface from FW Context A inside (VRF CMI) |
| <FW_CTXA_OUTSIDE_NEXTHOP> | FW Context A outside next-hop for cust_out default |
| <FW_CTXA_INSIDE_NEXTHOP> | FW Context A inside next-hop for CMI default |
| <FW_CTXB_OUTSIDE_E02_IP>/<MASK> | E02 interface to FW Context B outside |
| <FW_CTXB_INSIDE_E02_IP>/<MASK> | E02 interface from FW Context B inside (VRF CMI) |
| <FW_CTXB_OUTSIDE_NEXTHOP> | FW Context B outside next-hop for cust_out default |
| <FW_CTXB_INSIDE_NEXTHOP> | FW Context B inside next-hop for CMI default |
| <CUST_A_LINK_IP>/<MASK> | E01 interface toward Customer A CE |
| <CUST_B_LINK_IP>/<MASK> | E01 interface toward Customer B CE |
| <CUST_C_LINK_IP>/<MASK> | E02 interface toward Customer C CE |
| <ALLOWED_NETWORKS> | Permitted networks in CMI_FILTER ACL |

Reference: InterAS Option B HLD v3.0