InterAS Option B

Connectivity Design Document

Cisco Nexus 7000/7700 Series  |  NX-OS MPLS

March 2026

Based on Cisco NX-OS MPLS Configuration Guide (Release 8.x)

# 1. Overview

This document provides a comprehensive design for InterAS Option B connectivity using Cisco Nexus 7000/7700 series switches running NX-OS. It covers architecture, configuration, and verification for all three Option B variants supported on the platform.

InterAS Option B enables MPLS VPN traffic to flow between separate Autonomous Systems (AS) by establishing MP-eBGP sessions between Autonomous System Border Routers (ASBRs). Unlike Option A, which requires per-VRF sub-interfaces, Option B uses a single BGP session to exchange all labeled VPN prefixes, providing significantly better scalability.

## 1.1 Scope

This design covers three InterAS Option B variants:

InterAS Option B (Standard) – Full implementation using LDP between ASBRs for IGP label distribution (NX-OS 7.2(0)D1(1)+)

InterAS Option B with RFC 3107 – BGP labeled unicast replaces LDP for complete IGP isolation between data centers (NX-OS 7.2(0)D1(1)+)

InterAS Option B Lite – Restricted version with local VRF support only (NX-OS 6.2(2)+)

## 1.2 Prerequisites

1. MPLS license installed on all Nexus 7000/7700 switches

2. NX-OS version 7.2(0)D1(1) or later (for full Option B / RFC 3107)

3. ASBRs must be directly connected

4. MPLS feature set installed in the default VDC

# 2. Architecture

## 2.1 Network Topology

The following diagram shows the InterAS Option B topology with two Autonomous Systems connected via ASBRs:

## 2.2 How It Works

In an InterAS Option B deployment, the ASBRs exchange VPN routing information (routes and labels) over a single MP-eBGP session. The key principles are:

Within each AS, routing information is shared using iBGP (VPNv4/VPNv6 address family)

Between ASes, routing information is shared using eBGP with labeled VPN prefixes

ASBRs rewrite the next-hop and MPLS labels when forwarding NLRIs to iBGP neighbors

Data packets carry two labels: an IGP/Core label (directs to ASBR) and a VPN route label (directs to VRF)

## 2.3 Key Design Decisions

Option B provides better scalability than Option A because it requires only one BGP session to exchange all VPN prefixes between the ASBRs. It also supports Nonstop Forwarding (NSF) and Graceful Restart. However, because traffic between ASBRs is MPLS-labeled, IP-based QoS mechanisms cannot be applied at the inter-AS boundary and VRFs cannot be isolated at the ASBR level.

Important limitation: InterAS Option B is not supported with BGP confederation AS configurations.

# 3. Option B Variant Comparison

The following table compares the three InterAS Option B variants:

| Feature | Option B (Standard) | Option B (RFC 3107) | Option B Lite |
| --- | --- | --- | --- |
| Min NX-OS Version | 7.2(0)D1(1) | 7.2(0)D1(1) | 6.2(2) |
| Label Distribution (Inter-ASBR) | LDP (IGP + MPLS) | BGP Labeled Unicast (no LDP needed) | LDP |
| IGP Isolation | No (IGP runs between ASBRs) | Yes (complete isolation) | No |
| iBGP VPNv4/v6 | Supported within AS | Supported within AS | Limited (no MP-BGP L3VPN within AS) |
| eBGP VPNv4/v6 | Between ASBRs | Between ASBRs | Between ASBRs |
| Per-VRF iBGP Required | No | No | Yes |
| Scalability | High (single BGP session) | High (single BGP session) | Limited |
| NSF / Graceful Restart | Supported | Supported | Not specified |
| ACL Filtering Needed | No | Yes (filter LDP port 646) | No |
| Recommended For | Standard inter-DC MPLS VPN | ISP / multi-DC with full IGP isolation | Legacy or simple deployments |

# 4. Configuration Templates

## 4.1 InterAS Option B (Standard)

### 4.1.1 Switch Feature Enablement

Enable the required features on all participating switches:

configure terminal

install feature-set mpls

feature-set mpls

feature mpls ldp

feature mpls l3vpn

feature bgp

! Define VRF

vrf context VPN1

rd 100:1

address-family ipv4 unicast

route-target import 1:1

route-target export 1:1

copy running-config startup-config

### 4.1.2 BGP Configuration (DC Edge / ASBR)

Configure iBGP and eBGP VPNv4/v6 sessions on DC Edge switches:

configure terminal

router bgp 100

! eBGP to remote ASBR

neighbor 10.0.0.2

remote-as 200

address-family vpnv4 unicast

send-community both

! VRF configuration

vrf VPN1

address-family ipv4 unicast

exit

copy running-config startup-config

## 4.2 InterAS Option B (RFC 3107)

This variant uses BGP labeled unicast instead of LDP between ASBRs, providing complete IGP isolation between data centers.

### 4.2.1 Switch Feature Enablement

Same as standard Option B (see Section 4.1.1).

### 4.2.2 BGP Configuration (ASBR - e.g., ASBR2 in AS 200)

Configure eBGP VPNv4/v6 along with BGP labeled unicast:

configure terminal

router bgp 200

address-family ipv4 unicast

redistribute direct route-map loopback

allocate-label all

exit

! eBGP to ASBR1 - labeled unicast (RFC 3107)

neighbor 10.1.1.1

remote-as 100

address-family ipv4 labeled-unicast

exit

exit

! eBGP to ASBR1 - VPNv4 via loopback

neighbor 1.1.1.1

remote-as 100

update-source loopback0

ebgp-multihop 2

address-family vpnv4 unicast

send-community both

### 4.2.3 iBGP Configuration (DC Edge within AS)

configure terminal

router bgp 200

address-family ipv4 unicast

redistribute direct route-map loopback

allocate-label all

exit

neighbor 3.3.3.3

remote-as 200

address-family vpnv4 unicast

next-hop-self

send-community both

### 4.2.4 ACL to Filter LDP Between ASBRs

Since RFC 3107 uses BGP for label distribution, LDP must be filtered on the inter-ASBR interface (port 646 TCP/UDP):

configure terminal

ip access-list LDP

10 deny tcp any any eq 646

20 deny tcp any eq 646 any

30 deny udp any any eq 646

40 deny udp any eq 646 any

50 permit ip any any

exit

interface ethernet 2/20

mpls ip

ip access-group LDP in

ip access-group LDP out

end

## 4.3 InterAS Option B (Lite Version)

The lite version has limitations: aggregation switch supports only local VRFs; routes learned from iBGP peers are not sent to eBGP peers and vice versa; MP-BGP Layer 3 VPN does not work within an AS.

### 4.3.1 Interface Configuration

configure terminal

interface ethernet 2/1

description To other ASBR

ip address 10.0.0.1 255.255.255.0

no shutdown

exit

interface ethernet 2/2

description To CE

vrf member VPN1

ip address 10.0.2.1 255.255.255.0

ipv6 address 2001:DB8:1::1

no shutdown

### 4.3.2 BGP Configuration (Lite)

configure terminal

router bgp 100

neighbor 10.0.0.2

remote-as 200

address-family vpnv4 unicast

send-community both

vrf VPN1

address-family ipv4 unicast

exit

copy running-config startup-config

# 5. Verification Commands

Use the following commands to verify your InterAS Option B deployment:

| Command | Purpose |
| --- | --- |
| show bgp vpnv4 unicast | Display VPN routes from the BGP table |
| show bgp vpnv6 unicast | Display IPv6 VPN routes from BGP table |
| show bgp ipv6 unicast vrf <name> | Display BGP info for a VRF (6VPE) |
| show forwarding ip route vrf <name> | Display IP forwarding table for a VRF |
| show ip bgp vrf <name> | Display BGP information on a VRF |
| show ip route vrf <name> | Verify CE-to-CE reachability |
| show running-config bgp | Display running BGP configuration |
| show running-config vrf <name> | Display VRF running configuration |
| show vrf <name> interface <type> | Verify RD and interface for VRF |
| trace <dest> vrf <name> | Trace route within a VRF |

# 6. Design Recommendations

## 6.1 Variant Selection Guide

Choose RFC 3107 variant when you need complete IGP isolation between data centers. Preferred by most service providers.

Choose Standard Option B when IGP convergence between ASBRs is acceptable and simpler configuration is desired.

Avoid Option B Lite for new deployments. Only use if running NX-OS prior to 7.2(0)D1(1).

## 6.2 Best Practices

1. Always use send-community both on eBGP VPNv4/v6 sessions to propagate route targets between ASes.

2. For RFC 3107, apply LDP-filtering ACLs on both inbound and outbound directions of inter-ASBR interface.

3. Use next-hop-self on iBGP VPNv4/v6 sessions within the AS for proper label assignment.

4. Verify route targets (import/export) are consistent across both ASes for end-to-end VPN connectivity.

5. Test failover scenarios by verifying NSF and Graceful Restart behavior on ASBRs.

6. Note: InterAS Option B is not supported with BGP confederation AS configurations.

## 6.3 Licensing

MPLS Layer 3 VPN requires an MPLS license on all Cisco Nexus 7000/7700 switches. Refer to the Cisco NX-OS Licensing Guide for complete licensing details.

Reference: Cisco Nexus 7000 Series NX-OS MPLS Configuration Guide, Chapter 24 - InterAS Option B